Demos

- Demo 1

  - Open the calculator and create a new file

  - Site information tab: yes CSW permit, zip=55155, 3 acres impervious, 2 acres turf B soil

  - Schematic tab

    - Bioretention no underdrain, 1 acre impervious , 3000 ft2 areas, 1.35 feet depth, B soil, 48 hour drawdown

    - Bioretention with underdrain, 2 acre turf B soil, 1 acre impervious, underdrain not raised, not lined, 3000 ft2 for each area, overflow depth=1.5 feet, 2 foot media depth, 0.11 FC-WPt, 0.25 MP-FC, no tree, Mix A, no P test, no amendment, B soil (0.45 in/hr), 48 hr drawdown

  - Infiltration basin, 1 acre impervious, 4000 ft2 areas, 1.8 foot overflow depth, B soils, 48 hour drawdown

  - Results tab: Is the performance goal met? If not, how can it be met?

- Demo 2 – C and D soils

  - 50 acre site – 20 percent impervious; 40 acres residential; 5 acres commercial; 5 acres green space on D soil; meet the CSW permit (1 inch retention requirement)

  - 30 acres turf on D soil; 10 acres turf on C soil (0.2 in/hr)

  - Individual bioretention max size = 10000 ft2

  - Can the goal be met with a realistic BMP scenario? If not, how can we maximize volume and P retention?

- Demo 3 – Ultra-urban setting

  - 50 acre site – 90 percent impervious; all commercial/business

  - A soils (0.8 in/hr)

  - Can the retention goal be met with a realistic BMP scenario? If not, how can we maximize volume and phosphorus retention?

  - What BMPs work well for this scenario?

Features, warnings, restrictions, BMPs

- Site information tab

  - Zip code – rainfall data

  - Question about CSW permit – affects BMPs available

  - Default values for retention requirement, P and TSS concentrations

  - Must have impervious acres

- Summary information toolbar on left

- Warnings involve a change of a default condition (e.g. changing the P concentration)

- Restrictions prevent you from entering data (examples - drawdown requirement, green roof and permeable pavement areas, bioretention depth

- Help button or links within each BMP – go to Stormwater Manual

- Bioretention with underdrain

  - Elevating the underdrain

  - Can be lined

  - Can include a tree

  - Maximum 1.5 foot water depth

  - Media mixes C and D retain P; A and B leach P unless P content is <30 ppm

  - Adding an amendment to attenuate phosphorus

- Permeable pavement

  - The area of permeable pavement must be included in the impervious acreage for the BMP

  - The impervious:permeable pavement area ratio cannot exceed 5:1 (e.g. for 1 acre of impervious, must have at least = 8712 of permeable pavement)

  - Volume credit for water stored beneath drain and infiltration during drawdown time

  - An effective BMP for retaining runoff

- Harvest and reuse/cistern

  - Storage volume – ponds can store very large volumes, while cisterns typically are limiting to what volume can be retained

  - User-defined max irrigation rate can be 2 in/week on A soils. On other soils default is lesser of defined rate or PET

  - Offline systems – drained during winter

  - Can retain water for non-irrigation uses

- Tree trench

  - Field capacity minus wilting point is available to plants

  - More ET for larger trees

  - Will get a warning and lose credit if soil volume per tree is below the recommended value

  - Phosphorus crediting is same as for bioretention

  - Note the ET credit

- Green roof

  - Can have a conventional roof drain to a green roof, but the conventional roof area must be equal to or less than the green roof area

  - Maximum media depth is 4 inches

  - No phosphorus credit

- Disconnection BMP

  - Can’t be used if you are trying to meet the Construction SW permit

  - On Watershed tab, must enter permeable acres that will be used as effective pervious area

  - Difficult to meet retention requirement with this BMP

- Swales

  - Side slope is routed to a swale main channel or with underdrain – treat as a single BMP (e.g. match lengths)

  - Have all impervious acres go to side slope(s) and none to main channel

  - Ways to increase infiltration

    - Put in check dams

    - Put in bioretention base

    - Make swale longer

- Underground infiltration

  - Along with infiltration basin/trench, the most effective retention BMP for highly permeable soils (A soils)

  - Must either know two values or go outside calculator to a spreadsheet to calculate these 2 values (equations were too difficult to incorporate into the calculator)

  - Vp = underground pipe/storage volume

  - Am = area of engineered media

Demo 2 (low permeability)

- Harvest and reuse; 5 acres on D; 3000 ft3 storage

- 4 bioretention basins with raised underdrain on C soil; Mix C; include tree

- Route 10 acres C, 25 acres D, and 8 acres impervious; include check dams; route to bioretention with underdrain

- Green roof – 2 acres

- Can’t get the performance goal (about 87% there); 74% P removal and 92% TSS removal

Demo 3

- Green roofs – 2 acres

- Permeable pavement - 6 acres impervious, 2 of which are ppavement (87120 ft2)

- Bioretention – 3 acres A soil turf; 2 acres impervious; 5000 ft2

- Tree trench – 2 acres turf; 30 acres impervious; 10 acres media area (435600 ft2); 2 foot media depth; medium tree; 875 trees

- Underground infiltration – 5 acres impervious; Vp = just enough to meet required retention volume